Disclaimer Notice:

This document is intended only for the use of the addressee and may contain confidential, personal, and/or privileged information.

Any unauthorized disclosure is strictly prohibited.

COMPREHENSIVE WORK PLAN

For:

name RESIDENCE

Peter Handsor

Mega roofing and Exteriors | 143 Huggard Road, Calgary, Alberta T3Z 2C2

ENVELOPE WORK EXECUTION PLAN

Mega Roofing and Exteriors. would like to thank you for considering our company with completing your building envelope replacement project. The following is the work execution plan for the building envelope replacement for:

Client Information

GENERAL CONDITIONS / REQUIREMENTS

Mega Roofing and Exteriors will be furnishing the following as part of the General Requirements for the building envelope project:

- A Project Forman will provide Site Supervision and coordination for our scope of work.

- Workers Compensation Board coverage and Insurance will be provided for our scope of work.

- Work shall be in performed in accordance with the Occupation Health and Safety Act.

- Materials and Installation - Reference Standards:

- Alberta Building Code / National Building Code (latest edition)

- Canadian Standards Association

- Shingle Manufactures' written standards

- Alberta Roofing Contractors Association standards

(Manual on Good Roofing Practice: latest revisions and available online only.)

- Mega Roofing and Exteriors will Provide adequate property protection including tarps over decks, and gardens.

- General site clean-up for our own scope of work.

- See: APPENDIX A: Itemized Scope of What to Expect.

- See: APPENDIX C: Glossary of Terms.

ITEMIZED SCOPE OF WORK			ROOFING

- REMOVAL OF EXISTING ROOFING: (Slope Roof Areas Only)

- Remove and dispose of existing shingles, damaged underlayment, damaged eaves protection, roof/plumbing vents. Leaving any bonded underlayment to the sheathing products. (Existing step, wall, and chimney flashings to remain).

- Inspect roof wood decking for rotting/damage.

- DRIP & RAKE EDGE METAL FLASHING:

- Supply and install pre-coloured 26 ga. for standard roof or 26 ga. for upgraded roof, drip flashing along all eaves and rake edges.

- Deck flange to extend a minimum of 3” onto deck and maximum of 1.5” on face.

- Rake edge will be broken at 90° and extend 1.0” on the face. See, Typical Metal Flashing Details.

- VALLEY METAL:

- Supply and install new pre-coloured, 26 ga. valley flashing in all valleys.

- Roof slopes of 7/12 or more shall have “W” valleys incorporated. See, Typical Metal Flashing Details.

- Any metal found with kinks/damages are not to be incorporated into the installation and will be removed from site.

- EAVES and VALLEY PROTECTION (ICE & WATER):

- Supply and install a self-adhering Styrene-Butadiene-Styrene (SBS) Modified Bitumen Membrane 36” width along all low slope eaves sections and 36” on step slopes and center in all valleys.

- Acceptable approved SBS products to be installed as per manufacturers guidelines.

- SHINGLE UNDERLAYMENT:

- Supply and install a synthetic underlayment on the REMAINING roof surface as per manufacturers guidelines.

- Supply and install 2 layers of underlayment to be installed on roofs slopes lower than 4/12 and greater than 2/12.

- Acceptable approved SBS products to be installed as per manufacturers guidelines.

- ROOFTOP ATTIC VENTING:

- Supply and install roof top static vents to allow for proper attic ventilation.

- DRYER, FAN AND WATERLINE PLUMBING VENTS:

- Remove and replace all existing roof top dryer and fan exhaust vents with new pre-coloured gooseneck vents.

- Remove and replace waterline plumbing vents with new, neoprene plumbing boots/flanges installed as per manufacturers guidelines.

- A & B VENT BASE FLASHING & CAPS:

- A & B Vent base flashings and rain caps to remain existing.

- Storm collars to be sealed with Alsans Flashing or MS Detail applied as per manufacturers guidelines.

- ROOFING MATERIALS:

  - Field Shingles:

- Supply and install the shingles as per manufacturers specifications/guidelines using 6 nails per shingle.

- Valleys are to be open cut and top cropped to facilitate water flow.

- California valley style installations are not to be allowed nor is blocking or raking of shingles an acceptable practice.

  - Starter Shingles:

- Supply and install starter shingles along all eaves and gable roof edges as per manufacturers specifications/guidelines.

  - Hip & Ridge Capping:

- Supply and install capping along all ridges and hips as per manufacturers specifications/guidelines.

- Capping to have 4 nails per (2 on each side)

- FASTENERS:

- All vent flanges and exposed flashings to be fastened with weather tight screws complete with corrosion resistant, self-sealing neoprene washers.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- All eavestrough sections are to be cleaned in full upon completion of the project.

- TIME FRAME:

- Anticipated time to complete the sloped roofing portion is approximately: One (1) Day weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

- The manufacturers standard warranty.

ITEMIZED SCOPE OF WORK		STANDING METAL SEAM ROOFING

- DRIP EDGE METAL FLASHING:

- Supply and install pre-coloured 26 ga. drip flashing along all eaves.

- Deck flange to extend a minimum of 3” onto deck and maximum of 1.5” on face. See, Typical Metal Flashing Details.

- EAVES PROTECTION (ICE & WATER):

- Supply and install a self-adhering High Temp modified bitumen membrane 36” width along all low slope eaves sections.

- Acceptable approved products to be installed as per manufacturers guidelines.

- ROOF UNDERLAYMENT:

- Supply and install a synthetic felt underlayment on the REMAINING roof surface as per manufacturers guidelines.

- Supply and install 2 layers of underlayment to be installed on roofs slopes lower than 4/12 and greater than 2/12.

- Acceptable approved products to be installed as per manufacturers guidelines.

- ROOFTOP ATTIC VENTING:

- Supply and install roof top static vents to allow for proper attic ventilation.

- ROOFING MATERIALS:

- Supply and install Master 1236 1-1/4" 28-gauge standard colour, standing metal seam roofing.

- FASTENERS:

- All vent flanges and exposed flashings to be fastened with weather tight screws complete with corrosion resistant, self-sealing neoprene washers.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- TIME FRAME:

- Anticipated time to complete the sloped roofing portion is approximately: Two (2) Days weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

- The manufacturers standard warranty.

ITEMIZED SCOPE OF WORK		FLAT ROOFING

- CAP METAL FLASHING:

- Supply and install pre-coloured 26 ga. cap flashing along all parapet walls.

- INSULATION:

- Supply and install roof deck ridged insulation to a R-value of XXX.

- Acceptable approved products to be installed as per manufacturers guidelines.

- SLOPE PAKAGE:

- Supply and install a lope package to direct the water runoff to the roof drains or scuppers.

- Acceptable approved products to be installed as per manufacturers guidelines.

- ROOFING MATERIALS:

- Single-Ply Membranes

- EPDM (Ethylene Propylene Diene Monomer): A durable, black synthetic rubber membrane that's long-lasting, low-maintenance, and good for large areas.

- TPO (Thermoplastic Polyolefin): A heat-reflective white membrane, energy-efficient, durable, and popular for commercial buildings.

- PVC (Polyvinyl Chloride): A durable, flexible, and excellent waterproof option, often used for demanding applications.

- Bituminous Systems

- Modified Bitumen (Mod-Bit): Asphalt-based sheets with added polymers (APP or SBS) for flexibility, applied by torch, self-adhering, or hot mopping.

- Built-Up Roofing (BUR / Tar & Gravel): Layers of asphalt/tar alternated with reinforcing fabrics, topped with gravel for protection.

- Fluid-Applied Systems

- Spray Polyurethane Foam (SPF): Sprayed on as a liquid to form a seamless, highly insulating layer.

- Liquid Coatings: Acrylic, silicone, or urethane applied as a liquid to create a waterproof membrane, often over existing roofs.

- FASTENERS:

- All vent flanges and exposed flashings to be fastened with weather tight screws complete with corrosion resistant, self-sealing neoprene washers.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- TIME FRAME:

- Anticipated time to complete the sloped roofing portion is approximately: Two (2) Days weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

- The manufacturers standard warranty.

ITEMIZED SCOPE OF WORK	EAVESTROUGH, and DOWNSPOUT

- REMOVAL OF EXISTING Eavestrough, and Downspout:

- Remove and dispose of existing aluminum eavestrough and downspout.

- Inspect wood fascia for rotting/damage.

- Remove and replace wood fascia if/where required at a rate of $5.00/lf.

- PROVIDE ALL LABOUR, MATERIALS and equipment required to execute the following scope of work:

- Supply and install pre-coloured Five Inches (5") continuous aluminum eavestrough complete with steel hangers, prefabricated end caps and custom mitered corners.

- Supply and install pre-coloured 2 5/8" x 2 5/8" aluminum downspouts complete with elbows, straps and kick-outs at grade. Downspout kick-outs to be complete with up/down placement.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- TIME FRAME:

- Anticipated time to complete the soffit, fascia, eavestrough, and downspout portion of the project is approximately:

Two (2) Days weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

- The manufacturers standard warranty.

ITEMIZED SCOPE OF WORK	SOFFIT

- REMOVAL OF EXISTING Soffit:

- Remove and dispose of existing aluminum soffit, fascia, eavestrough, and downspout.

- Inspect wood fascia for rotting/damage.

- Remove and replace wood fascia if/where required at a rate of $5.00/lf.

- PROVIDE ALL LABOUR, MATERIALS and equipment required to execute the following scope of work:

- Supply and install pre-coloured aluminum soffit.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- TIME FRAME:

- Anticipated time to complete the soffit, fascia, eavestrough, and downspout portion of the project is approximately:

Two (2) Days weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

- The manufacturers standard warranty.

ITEMIZED SCOPE OF WORK	FASCIA

- REMOVAL OF EXISTING Fascia:

- Remove and dispose of existing fascia

- Inspect wood fascia for rotting/damage.

- Remove and replace wood fascia if/where required at a rate of $5.00/lf.

- PROVIDE ALL LABOUR, MATERIALS and equipment required to execute the following scope of work:

- Supply and install pre-coloured aluminum fascia complete with gable/fascia trim.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- TIME FRAME:

- Anticipated time to complete the soffit, fascia, eavestrough, and downspout portion of the project is approximately:

Two (2) Days weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

- The manufacturers standard warranty.

ITEMIZED SCOPE OF WORK			VINYL SIDING

- REMOVAL OF EXISTING DAMAGED VINYL SIDING:

- Remove and dispose of existing vinyl siding on the NORTH and WEST exposures.

- Inspect wood sheathing for rotting/damage.

- Remove and replace sheathing if/where required at a rate of $5.00/sq ft.

- BUILDING WRAP:

- Supply and Install Tyvek home wrap or equivalent, on all exposures, sealing all seams with Tuck Tape or equivalent.

- WALL DRIP EDGE and REVERSE WALL DRIP EDGE METAL FLASHING:

- Supply and install pre-coloured 29 ga. standard or 26 ga. for upgraded wall drip flashing and reverse wall drip edge along all windows, doors and horizontal trims on the exterior wall that are to receive new vinyl siding.

- Wall flange to extend a minimum of 3” onto the wall and sealed to the building envelope with a self adhered membrane installed as per manufacturers guidelines.

- Drip edge will be broken at 80°. See, Typical Metal Flashing Details.

- UNIT UTILITIES:

- Supply and install new exterior faucet, outlet boxes and light trim mounts as required installed as per manufacturers guidelines.

- VINYL SIDING:

- Supply and install vinyl siding installed as per manufacturers guidelines. (Standard colours with the closest possible match to the existing colour).

- All siding to have a minimum ¼” clearance from flashings, 2” from roofs or hard landscaping (concrete, asphalt, pavers, etc.), and 8” from soft landscaping (dirt, grass, mulch, or rock beds, etc.).

- CAULKING:

-	Supply and install caulking at all vertical joints of dissimilar materials.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- TIME FRAME: Anticipated time to complete the vinyl siding is approximately: One (1) Week weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

- The manufacturers standard warranty.

ITEMIZED SCOPE OF WORK		HARDIE SIDING - OPTION

- REMOVAL OF EXISTING DAMAGED VINYL SIDING:

- Remove and dispose of existing vinyl siding on ALL exposures.

- Inspect wood sheathing for rotting/damage.

- Remove and replace sheathing if/where required at a rate of $5.00/sq ft.

- BUILDING WRAP:

- Supply and Install Tyvek home wrap or equivalent, on all exposures, sealing all seams with Tuck Tape.

- DRIP EDGE and REVERSE DRIP EDGE METAL FLASHING:

- Supply and install pre-coloured 29 ga. standard or 26 ga. for upgraded drip flashing and reverse drip edge along all windows, doors and horizontal trim on the exterior wall that are to receive new Hardie siding.

- Wall flange to extend a minimum of 3” onto the wall and sealed to the building envelope with a self adhered membrane installed as per manufacturers guidelines.

- Drip edge will be broken at 80°. See, Typical Metal Flashing Details.

- WINDOW and DOOR BATTENS

- Supply and install 5 5/8” Hardie battens at all window and door heads, and sills with 3 5/8” jambs,

- All horizontal battens to have metal flashing both on the top and the bottom. Typical Metal Flashing Details.

- BELLY BAND BATTENS

- Supply and install 11 5/8” Hardie battens at all floor transitions.

- All horizontal battens to have metal flashing both on the top and the bottom. Typical Metal Flashing Details.

- CORNER BATTENS

- Supply and install 5 5/8” Hardie battens at all inside and outside corners.

- GABLE BATTENS

- Supply and install 5 5/8” Hardie battens at all gables.

- All gable battens to have metal flashing on the bottom. Typical Metal Flashing Details.

- UNIT UTILITIES:

- Supply and install new exterior faucet, outlet boxes and light trim mounts as required installed as per manufacturers guidelines.

- HARDIE BOARD SIDING:

- Supply and install Hardie siding with a 7” exposure installed as per manufacturers guidelines.

(Standard colours).

- All butt joints the have a 6”x10” Hardie strip installed behind.

- All siding to have a minimum ¼” clearance from flashings, 2” from roofs or hard landscaping (concrete, asphalt, pavers, etc.), and 8” from soft landscaping (dirt, grass, mulch, or rock beds, etc.).

- HARDIE SHAKE SIDING:

- Supply and install Hardie Shake Siding Straight with a 6" exposure with installed as per manufacturers guidelines.

(Standard colours).

- All butt joints the have a 6”x10” Hardie strip installed behind.

- All siding to have a minimum ¼” clearance from flashings, 2” from roofs or hard landscaping (concrete, asphalt, pavers, etc.), and 8” from soft landscaping (dirt, grass, mulch, or rock beds, etc.).

- HARDIE PANEL SIDING:

- Supply and install Hardie Panel Siding Smooth Panels ColorPlus with installed as per manufacturers guidelines.

(Standard colours).

- Supply and install vertical 2” Hardie battens at 12" on center all gables.

- All siding to have a minimum ¼” clearance from flashings, 2” from roofs or hard landscaping (concrete, asphalt, pavers, etc.), and 8” from soft landscaping (dirt, grass, mulch, or rock beds, etc.).

- CAULKING:

-	Supply and install caulking at all vertical joints of dissimilar materials.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- TIME FRAME: Anticipated time to complete the Hardie siding is approximately: Two (2) Weeks weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

- The manufacturers standard warranty.

Design your house in Hardie here:

https://www.jameshardie.com/hardie-designer/

ITEMIZED SCOPE OF WORK:			STUCCO

- REMOVAL OF EXISTING DAMAGED STUCCO:

- Remove and dispose of existing siding on ALL exposures.

- Inspect wood sheathing for rotting/damage.

- Remove and replace sheathing if/where required at a rate of $5.00/sq ft.

- BUILDING WRAP:

- Supply and Install Tyvek home wrap or equivalent, on all exposures, sealing all seams with Tuck Tape.

- DRIP EDGE and REVERSE DRIP EDGE METAL FLASHING:

- Supply and install pre-coloured 29 ga. standard or 26 ga. for upgraded drip flashing and reverse drip edge along all windows, doors and horizontal trim on the exterior wall that are to receive new Stucco.

- Wall flange to extend a minimum of 3” onto the wall and sealed to the building envelope with a self adhered membrane installed as per manufacturers guidelines.

- Drip edge will be broken at 80°. See, Typical Metal Flashing Details.

- STUCCO:

- Supply and install Stucco installed as per manufacturers guidelines.

- All stucco to have a minimum ¼” clearance from flashings, 2” from roofs or hard landscaping (concrete, asphalt, pavers, etc.), and 8” from soft landscaping (dirt, grass, mulch, or rock beds, etc.).

- CAULKING:

-	Supply and install caulking at all vertical joints of dissimilar materials.

- FINAL CLEANUP OF SITE:

- A final, thorough cleanup will be completed to ensure all materials, debris etc. are removed from the site completely.

- TIME FRAME: Anticipated time to complete the Stucco is approximately: Two (2) Weeks weather dependant.

- WARRANTIES/GUARANTEES:

Upon completion of the work, Mega Roofing & Exteriors Inc. will provide the following warranties:

- Standard 5-year labor/workmanship warranty.

The manufacturers standard warranty.

ITEMIZED SCOPE OF WORK NOT COVERED	EXCLUSIONS

Mega Roofing and Exteriors will not undertake this project without the following exclusions addressed.

In order to install Hardie products they need to be attached to the building structure, studs or minimum 7/16” sheathing, or to furring that has been attached to the structure. To confirm where the composition of the structure, the lap siding, or the rock dash stucco needs to be removed.

- ASBESTOS TESTING:

- Based on the age of construction, circa 1955, the following building components typically contained asbestos:

- The paint on the original lap siding. (Asbestos in paint was discontinued in the late 1970s)

- The sealants around the doors and windows at the original cladding layer. (Asbestos in sealants was effectively phased out of most construction materials by the 1990s.)

- The stucco. (Asbestos in stucco was discontinued in the 1990s)

- Asbestos testing is not included in the quote for this project.

- Asbestos testing to be conducted by others qualified to do this work.

- ABATEMENT:

- In the event that asbestos if found in the paint, sealants, or the stucco, an abatement will be required.

- Abatement testing is not included in the quote for this project.

- Abatement to be conducted by others qualified to do this work.

- BUILDING SHEATHING:

- The building sheathing needs to be exposed to confirm that it meets the specifications required for the installation of Hardie products. Based on the age of construction, circa 1955, the buildings may be sheathed in Buffalo Board. This product is inadequate for the installation Hardie products.

- Removal of the Buffalo Board is not included in the quote for this project.

- The supply and installation of minimum 7/16” sheathing is not included in the quote for this project.

- BUILDING STRUCTURE:

- The building structure may have been subjected to moisture infiltration over the approximate 70-year life span of the buildings. As a result, there may be structural damage to the 2x4 studs and remediation work may be required.

- The supply and installation of new 2x4 studs is not included in the quote for this project.

- BUILDING INSULATION:

- The building insulation may have been subjected to moisture or rodent infiltration over the approximate 70-year life span of the buildings. As a result, there may be damaged or missing insulation and remediation work may be required.

- The supply and installation of new insulation is not included in the quote for this project.

TYPICAL METAL FLASHING DETAILS

4”

4”

3”

1 ½”

½”

| Not to scale 5” ½” 6” ½” | Not to scale 1” 4” ½” | Not to scale 12” Minimum 12” Minimum |
| --- | --- | --- |
| CAP FLASHING Pre-Coloured Galvanized Steel 29 ga. for standard roof or 26 ga. for upgraded roof. | SKYLIGHT COUNTER FLASHING Pre-Coloured Galvanized Steel 29 ga. for standard roof or 26 ga. for upgraded roof. | VALLEY FLASHING Pre-Coloured Galvanized Steel 26 ga. |
| Not to scale | Not to scale 3” 1” ½” | 24” Not to scale With 1” Center Ridge Roof pitches of 7:12 and more. |
| EAVES DRIP EDGE Pre-Coloured Galvanized Steel 29 ga. for standard roof or 26 ga. for upgraded roof. | RAKE DRIP EDGE Pre-Coloured Galvanized Steel 29 ga. for Standard roof or 26 ga. for upgraded roof. | “W" VALLEY FLASHING Pre-Coloured Galvanized Steel 26 ga. |
| Not to scale 3” 110° ½” ½” Trim Depth + ¼” | Not to scale | 80° ½” 3” ½” Trim Depth + ¼” |
| WALL and STEP FLASHING Pre-Coloured Galvanized Steel 29 ga. for standard roof or 26 ga. for upgraded roof. | HEAD or WALL FLASHING DETAIL Pre-Coloured Galvanized Steel 29 ga. standard or 26 ga. for upgraded. | Not to scale WINDOWSILL FLASHING DETAIL Pre-Coloured Galvanized Steel 29 ga. standard or 26 ga. for upgraded. |

PROJECT PRICING

ROOFING SYSTEM - 30 Year Roof:

- GAF Timberline HDZ:	$.00 + GST

Initial here to

accept Option 1.

(GAF Weatherblocker Starter Strip, GAF Seal-A-Ridge, GAF Leak Protector 36",

TriBuilt Synthetic, Drip Edge - 29 ga, Rake Edge - 29 ga, Valley Flashing 26 ga,

Primex Goosenecks, WeatherPRO50, and 4" Oatley)

- GAF Registered Lifetime Warranty: 	$150.00 + GST

Initial here to

accept.

OPTIONAL ROOF UPGRADES and PRICING:

CLASS 4 ROOF - 40 Year Roof:

Initial here to

accept Option 2.

Malarkey Vista:	$.00 + GST

(Malarkey Smart Start, Malarkey EZ Ridge XT 10", Malarkey 401 Arctic Seal 36"

Malarkey 1030 SecureStart SG, Drip Edge - 26 ga - 1.5" Spec, Rake Edge - 26 ga - 1" Spec.

Valley Flashing - 26 ga, Primex Goosenecks, WeatherPRO50, and 4" Oatley)

CLASS 4 ROOF - 50 Year Roof:

Initial here to

accept Option 3.

Malarkey Legacy:	$.00 + GST

(Malarkey Smart Start, Malarkey EZ Ridge XT 10", Malarkey 401 Arctic Seal 36"

Malarkey 1030 SecureStart SG, Drip Edge - 26 ga - 1.5" Spec, Rake Edge - 26 ga - 1" Spec.

Valley Flashing - 26 ga, Primex Goosenecks, WeatherPRO50, and 4" Oatley)

RUBBER ROOF - 50 Year Roof:

Initial here to

accept Option 3.

Euroshield Beaumont HP: (STANDARD COLOURS)	$.00 + GST

(Euroshield Starter, Euroshield Hip and Ridge, Resisto LB1236 36" (Euroshield)

FT Synthetics Platinum (Euroshield), Drip Edge - 26ga - 1.5" Spec, Rake Edge - 26ga - 1" Spec.

W Valley Flashing - 26ga, VMAX 303 Vent (appx. 16), and 4" Oatley)

RUBBER ROOF - 50 Year Roof:

Initial here to

accept Option 3.

Euroshield Beaumont HP: (Driftwood)	$.00 + GST

(Euroshield Starter, Euroshield Hip and Ridge, Resisto LB1236 36" (Euroshield)

FT Synthetics Platinum (Euroshield), Drip Edge - 26ga - 1.5" Spec, Rake Edge - 26ga - 1" Spec.

W Valley Flashing - 26ga, VMAX 303 Vent (appx. 16), and 4" Oatley)

STANDING METAL SEAM ROOFING SYSTEM:

Initial here to

accept this Option.

Standing metal seam for the entire roof:	$.00 + GST

FLAT ROOFING SYSTEM:

Initial here to

accept this Option.

Flat Roof areas indicated in the drawing DETAIL PLAN:	$.00 + GST

EAVESTROUGH and DOWNSPOUT SYSTEM:

- 5" Aluminum Eavestrough with 2 5/8" x 2 5/8" downspouts:	$.00 + GST

Initial here to

accept this Option.

OPTIONAL EAVESTROUGH and DOWNSPOUT UPGRADES AND PRICING:

Initial here to

accept this Option.

6" Aluminum Eavestrough with 3" x 3" downspouts:	$.00 + GST

VINYL CLADDING SYSTEM:

Initial here to

accept this Option.

Vinyl Cladding on the NORTH and WEST exposures:	$.00 + GST

Initial here to

accept this Option.

Vinyl Cladding on the WHOLE HOUSE OPTION:	 $.00 + GST

(In the event that the colour has been discontinued)

HARDIE CLADDING SYSTEM OPTION:

Initial here to

accept this Option.

Hardie Cladding on the WHOLE HOUSE:	$.00 + GST

UPGRADED FLASHINGS OPTION:

Initial here to

accept this Option.

26 ga. Throughwall, Head and Windowsill Flashings:	$.00 + GST

RAIN SCREEN OPTION:

Initial here to

accept this Option.

Add Rainscreen Furring and Insulation Upgrade:	$.00 + GST

- Supply and install 1” x 3” vertical furring to provide a rainscreen configuration for the siding installation.

- Supply and install IKO EnerAir 1" Insulation.

- Furring to be attached to solid structure.

WINDOW SCREEN REPAIRS:

- Window Screen Repairs NORTH and WEST exposures:	(Requires Repair)

- Small- Roughly 2ft x 2ft- $43.00 plus tax per screen

- Medium- Roughly 3ft x 3ft- $55.00 plus tax per screen

- Large- Roughly 4ft x 4ft- $96.00 plus tax per screen

- X-Large- Roughly 5ft x 5ft- $149.00 plus tax per screen

PLEASE SIGN THE FORM BELOW

I hereby give access to the above-mentioned property and authorization to Mega Roofing and Exteriors Inc. for the purpose of undertaking the assigned scope of work outlined in the attached Estimate dated April 24, 2026.

I hereby acknowledge that I am responsible for payment to Mega Roofing and Exteriors Inc. in the amount of:

$			.00+GST upon completion of the scope of work.

Buyer’s Right to Cancel

You may cancel this contract from the day you enter into the contract until 10 days after you receive a copy of the contract. You do not need a reason to cancel.

If you do not receive the goods or services within 30 days of the date stated in the contract, you may cancel this contract within one year of the contract date. You lose that right if you accept delivery after the 30 days. There are other grounds for extended cancellation. For more information, you may contact your provincial/territorial consumer affairs office.

If you cancel this contract, the seller has 15 days to refund your money and any trade-in, or the cash value of the trade-in. You must then return the goods.

To cancel, you must give notice of cancellation at the address in this contract. You must give notice of cancellation by a method that will allow you to prove that you gave notice, including registered mail, fax or by personal delivery.

***NOTE: All pricing is valid for 30 days***

ACCEPTANCE:

|  | Mega Roofing & Exteriors Inc. |  |  | Owner: |
| --- | --- | --- | --- | --- |
| Name: | Peter Handsor |  | Name: |  |
| Title: | Project Manager / Estimator |  | Title: |  |
| Sign: | Peter Handsor |  | Sign: |  |
| Date: | April 24, 2026 |  | Date: | 2026 |

Shingle Selection and Colour:

Eavestrough and Downspout Selection and Colour:

Siding Selection and Colour (s):

***NOTE: Payment is due on completion***

ROOF PLAN

N

HAIL ASSEMENT PHOTOS

The photos included in this document are not exhaustive of all the damage associated with the building but rather a visual representation of each type of damage observed.

| COMMENTS: | Image Number 1 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 2 (Source: Mega 2026) |

| COMMENTS: | Image Number 3 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 4 (Source: Mega 2026) |

| COMMENTS: | Image Number 5 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 6 (Source: Mega 2026) |

| COMMENTS: | Image Number 7 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 8 (Source: Mega 2026) |

| COMMENTS: | Image Number 9 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 10 (Source: Mega 2026) |

| COMMENTS: | Image Number 11 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 12 (Source: Mega 2026) |

| COMMENTS: | Image Number 13 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 14 (Source: Mega 2026) |

| COMMENTS: | Image Number 15 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 17 (Source: Mega 2026) |

| COMMENTS: | Image Number 18 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 19 (Source: Mega 2026) |

| COMMENTS: | Image Number 20 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 21 (Source: Mega 2026) |

| COMMENTS: | Image Number 22 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 23 (Source: Mega 2026) |

| COMMENTS: | Image Number 24 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 25 (Source: Mega 2026) |

| COMMENTS: | Image Number 26 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 27 (Source: Mega 2026) |

| COMMENTS: | Image Number 28 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 29 (Source: Mega 2026) |

| COMMENTS: | Image Number 30 (Source: Mega 2026) |
| --- | --- |
| COMMENTS: | Image Number 31 (Source: Mega 2026) |

APPENDIX A 				ITEMIZED SCOPE OF WHAT TO EXPECT

Thank you for taking the time to review this letter - we are Mega Roofing & Exteriors Inc. (Mega), and we are happy to be working with you to complete the building envelope project. Here are a few things to know and expect during the process:

- While Mega. is onsite, completing the roofing and or exterior work, the area / complex will be regarded as a construction / work site. As such, the residents are not permitted on the roofs during roofing work and shall refrain from climbing the ladders and or scaffolding.

- The residents are expected to take the utmost care while driving or walking through the complex. Mega. will do our best to flag off hazards but request that you exercise critical thinking.

- The work being undertaken can be quite messy so please except to find some debris in the gardens, on lawns, on rear decks, on sidewalks and driveways. Mega is expected to clean this up the best we can throughout the day, by the end of each workday and a final walkthrough at completion.

- Please remove all furniture, barbecues, etc. from your deck. If you are unable to move these items, the crew would be pleased to assist.

- Nails are usually the biggest problem on a roofing and or exterior work job site as they can be harmful to pets, humans, and vehicle tires. Everyday Mega’s crew will run a magnet around the property to pick up nails and fasteners, but they can be missed.  Please be vigilant, Mega will not cover the cost for the replacement of ‘run flat’ tires.

- During the roofing and or exterior work replacement program hammering, banging and vibrations will occur due to the work. Wall furnishings, paintings, chandeliers, and other breakable items that may be affected by the vibrations should be removed and stored, until all roofing and or exterior work is completed. Mega will do their best to minimize these effects.

- It is important to note, that there will be a large variance of weight on the roofs and vibrations, which can result in hair line fractures and or nail pops in the interior finish. Mega. will take precautions to avoid this however, the damages may be unavoidable. Mega cannot be held responsible for these damages, and they should be considered an inherent risk of the roof replacement program.

- Satellite dishes that require removal to complete the roofing and or exterior work will be reinstalled/mounted however, repositioning of the satellites for signal strength shall not be Mega’s responsibility.

- Mega will require the use of water and power for small hand tools and air compressors during the roofing and or exterior work. We thank you for making that readily available to us.

- The crews will be starting early in the morning and working, sometimes, late into the afternoon or early evening. If there are special circumstances in your home, please advise the board representative and we will do our best to accommodate you, although it may not always be possible.

- Please be assured that we will do our best to minimize noise and the length of time we are working on your home.

If you have specific roofing / exterior questions for us, please call or email your Project Manager:

APPENDIX C 				GLOSSARY OF TERMS

ROOFING

A & B Vent - A & B Vent (Type B Vent) refers to a double-walled metal chimney pipe system used for venting exhaust gases from specific gas appliances like furnaces, water heaters, and some fireplaces, drawing combustion air from the room while exhausting fumes through the roof; it's a common, relatively safe, but less efficient option than sealed systems because it uses indoor air.

Attic Ventilation - Attic ventilation is a balanced system of intake (soffit vents) and exhaust (ridge/gable vents) that creates airflow, removing hot, moist air in summer and preventing condensation/ice dams in winter, protecting your roof and improving energy efficiency by reducing AC load and preserving shingles. Proper systems use convection (hot air rises) for natural flow, requiring low intake vents and high exhaust vents, ideally with baffles to keep insulation from blocking airflow.

Drip - A drip edge is an L-shaped metal flashing installed along roof edges (eaves, rakes) to direct rainwater into gutters or away from fascia, preventing water from seeping under shingles, rotting wood, damaging the roof deck, and deterring pests, now often required by building codes for essential water management and a finished look. It ensures proper water runoff, protects the fascia board, and seals gaps for better roof longevity, especially during wind-driven rain or icy conditions.

Eaves - Eaves are the overhanging edges of a roof that extend beyond the walls of a building, primarily designed to direct rainwater away from the structure, protecting siding, windows, and foundations, while also providing shade and influencing attic ventilation and aesthetics. They can be decorative, finished with soffits, or left open, with components like fascia and gutters often integrated for a complete look and function, though they require maintenance and can attract pests.

Gooseneck Vents - Gooseneck vents are specialized roof vents with a curved, elevated design, used to safely exhaust moisture and heat from dryers, kitchen fans, or bathrooms, preventing water/snow/pests from entering while allowing continuous outward airflow. Made from materials like galvanized steel or aluminum, they feature internal dampers and screens, with sizes and finishes available for various roof types and aesthetics, ensuring weather protection for building exhaust system.

High Temp Modified Bitumen Membrane - A High Temp modified bitumen membrane uses polymers (like APP or SBS) to make asphalt durable for flat/low-slope roofs, with "High Temp" versions specifically designed for intense heat (up to 257°F/125°C) and UV resistance, often self-adhering (self-adhered), used under metal/tile roofs, offering superior waterproofing, durability, and resisting cracking in temperature swings.

Ice and Water - Ice and water barriers are your best defense for preventing water leaks caused by ice dams. Ice and water barriers are: Self-Sealing, preventing water infiltration around nails. Waterproof, giving added protection to valleys and roof openings. Self-Adhesive, allowing no space for water migration.

Ice Dams - An ice dam is a ridge of ice that forms at the edge of a roof, preventing melting snow from draining, causing water to back up, seep under shingles, and leak into walls, ceilings, and insulation, leading to significant home damage. They form from uneven roof temperatures (warm attic/cold eaves), causing snow to melt, run down, and refreeze at the colder edge, creating a dam that traps water, which can cause leaks. Prevention focuses on attic insulation, ventilation, and sometimes roof raking or heat cables, while removal involves safe methods like using pantyhose with ice melt or professional help.

Rake – A rake edge is the sloping side that frames the peak and gives your house that classic “A-frame” look. It's more than just a decorative element, it serves a primary function by sealing and protecting the most exposed area of your roof system.

Plumbing Boots/Flanges - Plumbing boots (or flanges/jacks) are essential roof components that create a watertight seal around vent pipes, preventing leaks where pipes penetrate the roof deck, using materials like flexible rubber/EPDM or durable metal with integrated flashing to accommodate different pipe sizes and roof pitches (e.g., Perma-Boot, standard rubber boots). They are installed with sealant and fasteners, often layered under shingles or integrated with flashing tape, to direct water away and protect against UV/weather damage for years.

Shingle Underlayment - Shingle underlayment is a water-resistant layer (like felt or synthetic material) installed directly on the roof deck, beneath shingles, acting as a crucial secondary barrier to protect your home from leaks, mold, and water damage if shingles fail or during extreme weather, creating a smoother surface and adding durability to the entire roofing system.

Sloped Roof - A sloped roof (or pitched/angled roof) is a building structure with an incline, designed to shed water and snow efficiently, preventing pooling and damage, common in homes with materials like shingles or metal, and defined by its pitch (rise over run, e.g., 4:12) which varies by climate and style, creating attics or vaulted ceilings.

Styrene-Butadiene-Styrene - Styrene Butadiene Styrene (SBS) is a versatile thermoplastic elastomer, meaning it acts like rubber but can be processed like plastic, combining properties of both: elasticity at room temp, flow when heated, great for adhesives, asphalt modification (roads/roofing), footwear, and compounding plastics due to its durability and impact strength. It's a block copolymer made from styrene and butadiene monomers, offering good elasticity, processability, and low-temp toughness, distinct from SBR (Styrene-Butadiene Rubber), which is a different polymer.

Valley - A roof valley is the V-shaped internal channel where two sloping roof sections meet, crucial for directing large amounts of rainwater and snowmelt efficiently to the gutters, preventing leaks and water damage by channeling runoff away from vulnerable roof areas. Valleys are essential but are high-risk spots for leaks if not installed correctly, often requiring metal flashing and specific shingle techniques like closed-cut or open valleys to ensure watertightness and longevity. W-Valleys are recommended for roofs with slopes greater than 4:12.

W-Valleys - W-Valleys are a type of metal roof flashing shaped like an inverted "W" with a central rib, used in roof valleys where two slopes meet to efficiently channel water and debris downward, preventing leaks and keeping the area protected, common in metal, slate, and shingle roofing for superior water management.

SOFFIT, FASCIA, EAVESTROUGH, and DOWNSPOUT

Downspout - A downspout (or downpipe) is the vertical pipe that channels rainwater from your roof's eavestrough down to the ground or drainage system, preventing water from damaging your home's foundation, siding, and landscaping by directing it away from the building. They come in various materials like aluminum, vinyl, steel, or copper and different shapes (round, rectangular) to match your home's aesthetics and drainage needs, often with elbows and leader (extensions) to guide water further away from the structure.

Downspout Leader - In roof drainage, a downspout leader (often simply called a "leader") is the horizontal pipe that channels rainwater from upper roof eavestroughs down to lower roof eavestroughs, or along the ground away from the building, or into a storm drainage system.

Eavestrough - An eavestrough (or gutter) is a crucial part of a home's water management system, forming a trough along the roof's edge to collect and channel rainwater and snowmelt via downspouts away from the foundation, preventing structural damage, basement flooding, and erosion of landscaping. Typically made from aluminum, steel, or vinyl, eavestroughs protect siding, roofs, and foundations, with seamless options and leaf guards available for better function and less maintenance.

Fascia - A fascia is a long, horizontal board installed along the roof's edge (eaves), directly attached to the ends of rafters or trusses, serving as a decorative trim that protects the roof structure from weather, provides a nailing surface for eavestroughs, and covers exposed rafter tails for a finished look, often made from wood, composite, or PVC materials.

Soffit - A soffit is the finished, horizontal surface covering the underside of a roof's overhang or eave, connecting the roofline to the exterior wall, providing essential ventilation to the attic, protecting rafters from moisture and pests, and offering a clean, architectural look in materials like vinyl, aluminum, or wood. Soffits can be vented (perforated) or solid, crucial for preventing heat/moisture buildup, mold, and insect infestations in your attic.

Splash Pad - A downspout splash pad is a product placed at the base of a downspout to divert rainwater away from the foundation of a building, preventing soil erosion and potential water damage. These are available in various materials and styles.

SIDING

Battens - These are vertical strips of material used in "board and batten" siding. They are applied over the seams of the wider boards or panels to provide weather protection and a distinct aesthetic.

Belly Band - This is a horizontal trim piece that creates a visual break and transition between different types or rows of siding materials on a building's exterior. It is often used to cover the horizontal seam where two panels meet on a tall wall, defining the home's proportions and adding an architectural look.

Revers wall drip edge - A wall drip edge is an inverted U-shaped metal flashing installed along windows sills, to direct rainwater outside the cladding or away from the building. Now often required by building codes for essential water management and a finished look. It ensures proper water runoff and protects the cladding.

Wall drip edge - A wall drip edge is an L-shaped metal flashing installed along windows, doors, bellybands, and at changes of cladding type to direct rainwater outside the cladding or away from the building. Now often required by building codes for essential water management and a finished look. It ensures proper water runoff and protects the cladding.